LINE FOLLOWING CAR CODE

.syntax unified

.cpu cortex-m3

.thumb

.global Reset_Handler

.global main

.section .text

Reset_Handler:

LDR   SP, =_estack       // Initialize stack pointer

BL    main

B     .

main:

// Enable clocks for GPIOA and GPIOB

LDR   R0, =0x40021018     // RCC_APB2ENR

LDR   R1, [R0]

ORR   R1, R1, #(1 << 2)   // IOPAEN

ORR   R1, R1, #(1 << 3)   // IOPBEN

STR   R1, [R0]

// Configure PA3-PA6, PA8-PA11 as Output Push-Pull (motors and ultrasonic TRIG)

LDR   R0, =0x40010800     // GPIOA_CRL

MOV   R1, #0x22222222     // MODE = 0b10 (Output), CNF = 0b00 (Push-pull)

STR   R1, [R0]

LDR   R0, =0x40010804     // GPIOA_CRH (PA8-PA11)

MOV   R1, #0x22222222

STR   R1, [R0]

// Configure PB0, PB1 as Input floating (IR sensors)

LDR   R0, =0x40010C00     // GPIOB_CRL

MOV   R1, #0x44444444

STR   R1, [R0]

BL    loop

loop:

// Read ultrasonic distance

BL    getDistance

// Compare distance <= 10

CMP   R0, #10

BLE   stopMotors

// Read IR sensors PB0 and PB1

LDR   R1, =0x40010C08     // GPIOB_IDR

LDR   R2, [R1]

ANDS  R3, R2, #1          // LEFT_IR = PB0

ANDS  R4, R2, #2          // RIGHT_IR = PB1

CMP   R3, #0

BNE   check_left_right

CMP   R4, #0

BNE   check_left_right

BL    moveForward

B     loop

check_left_right:

CMP   R3, #0

BNE   check_right

CMP   R4, #2

BNE   stopMotors

BL    turnRight

B     loop

check_right:

CMP   R3, #1

BNE   stopMotors

CMP   R4, #0

BNE   stopMotors

BL    turnLeft

B     loop

stopMotors:

LDR   R0, =0x4001080C     // GPIOA_BSRR

LDR   R1, =0x00F00000     // Clear IN1-IN4 (PA3â€“PA6)

STR   R1, [R0]

LDR   R0, =0x40010810     // GPIOA_ODR

LDR   R1, [R0]

BIC   R1, R1, #(1 << 8)   // Clear ENA (PA8)

BIC   R1, R1, #(1 << 9)   // Clear ENB (PA9)

STR   R1, [R0]

BX    LR

moveForward:

LDR   R0, =0x4001080C

LDR   R1, =((1<<2)|(1<<4)|(1<<(3+16))|(1<<(5+16))) // IN2 & IN3 HIGH, IN1 & IN4 LOW

STR   R1, [R0]

BX    LR

turnLeft:

LDR   R0, =0x4001080C

LDR   R1, =((1<<3)|(1<<5)|(1<<(2+16))|(1<<(4+16))) // IN1 & IN3 HIGH, IN2 & IN4 LOW

STR   R1, [R0]

BX    LR

turnRight:

LDR   R0, =0x4001080C

LDR   R1, =((1<<2)|(1<<4)|(1<<(3+16))|(1<<(5+16))) // IN2 & IN4 HIGH, IN1 & IN3 LOW

STR   R1, [R0]

BX    LR

getDistance:

// Send TRIG pulse on PA10

LDR   R0, =0x4001080C     // GPIOA_BSRR

LDR   R1, =(1 << 10)

STR   R1, [R0]

BL    delay10us

LDR   R1, =(1 << (10 + 16))

STR   R1, [R0]

// Wait for ECHO high on PA11

wait_echo_high:

LDR   R0, =0x40010808     // GPIOA_IDR

LDR   R1, [R0]

TST   R1, #(1 << 11)

BEQ   wait_echo_high

// Measure duration while ECHO is high (basic loop count)

MOV   R2, #0

wait_echo_low:

LDR   R1, [R0]

TST   R1, #(1 << 11)

BEQ   done_measure

ADD   R2, R2, #1

CMP   R2, #30000

BHI   done_measure

B     wait_echo_low

done_measure:

MOV   R0, R2

BX    LR

delay10us:

MOV   R2, #720          // Approx 10us delay

loop_delay:

SUBS  R2, R2, #1

BNE   loop_delay

BX    LR