COURSE PROJECT REPORT

Spring 2025

LINE FOLLOWING CAR

SUBMITTED BY:

Bilal Naeem (F23CSC008)

Sadia Tehreem (F23CSC043)

Saba Ramzan (F23CSC025)

FACULTY NAME Ms. SAADIA KARIM

Ms. SEEMA HASHWANI

A project report submitted in partial fulfilment of the Requirements for the award of the degree of Bachelor of Computer Science

COMPUTER ORGANIZATION AND ASSEMBLY LANGUAGE

DEPARTMENT OF COMPUTER SCIENCE

FACULTY OF INFORMATION TECHNOLOGY

FATIMA BUSINESS SCHOOL

SALIM HABIB UNIVERSITY, KARACHI

Date: 25-May-2025

#### DECLARATION

We hereby declare that this project report is based on our original work except for citations and quotations which have been duly acknowledged. We also declare that it has not been previously and concurrently submitted for any other degree or award at Salim Habib University or other institutions.

Name: Bilal Naeem		    Name: Sadia Tehreem		    Name: Saba Ramzan

Reg. No: F23CSC008		    Reg. No: F23CSC043		    Reg. No: F23CSC025

Signature:			    Signature: 				    Signature:

Date: 				    Date:				    Date:

#### APPROVAL FOR SUBMISSION

We certify that this project report entitled “Line Following Car” by “BILAL NAEEM, SADIA TEHREEM and SABA RAMZAN” has met the required standard for submission in partial fulfilment of the requirements for the award of Bachelor of Computer Science at Salim Habib University.

Approved by,

Supervisor: Ms Saadia Karim 	Co-Supervisor : Ms Seema Hashwani

Date: _________________________                Date:   _________________________

Signature: _____________________              Signature: _________________________

The copyright of this report belongs to Salim Habib University according to the Intellectual

Property Policy of Salim Habib University amended on July 2nd 2021.  Due acknowledgement shall always be made of the use of any material contained in, or derived from, this report.

© 2024 Salim Habib University. All rights reserved.

#### ACKNOWLEDGEMENTS

We would like to thank everyone who had contributed to the successful completion of this project. We would like to express my gratitude to my research supervisor, Miss Saadia Karim and Miss Seema Hashwani for their invaluable advice, guidance and his/her enormous patience throughout the development of the research.

In addition, we would also like to express my gratitude to our loving parent and friends who had helped and given me encouragement.

Yours Sincerely,

Bilal Naeem

Sadia Tehreem

Saba Ramzan

#### ABSTRACT

This project explores the automation of coal mining processes using advanced sensor integration and algorithmic control. Designed to enhance efficiency, reduce human risk, and improve operational accuracy, this initiative employs real-time monitoring systems and autonomous vehicles for mining tasks. The system integrates IR sensors, microcontrollers, and motor control algorithms to ensure precise operation and adaptability to complex environments.

Focusing on safety and operational efficiency, this project aims to address key challenges in traditional coal mining methods. By integrating cutting-edge sensor technologies and autonomous control, the system minimizes human involvement in hazardous tasks, thereby reducing risks while ensuring consistent performance. The inclusion of real-time monitoring further enhances adaptability to dynamic mining conditions, paving the way for smarter and safer mining practices.

Moreover, the project highlights the transformative role of automation in industrial settings, offering significant insights into the practical applications of sensor-based navigation and control. By leveraging modern technologies, this initiative not only enhances mining efficiency but also sets a precedent for innovation in resource extraction industries, contributing to the broader advancement of industrial automation.

### Table of Contents

- Introduction .................................................................................................. 1
   1.1 Background ............................................................................................... 1
   1.2 Problem Statements .................................................................................. 2
   1.3 Aims and Objectives ................................................................................. 3
   1.4 Scope of Project .................................................................................................... 3

- Literature Review ……………..................................................................................... 5
   2.1 Automation in Navigation Systems ....................................................... 5
   2.2 Role of Sensors in Line Following Cars ................................................. 6
   2.3 Microcontrollers and Real-Time Control ............................................... 7
   2.4 Challenges in Building Line Following Cars .......................................... 8
   2.5 Innovations and Future Trends ............................................................................. 9

- Project Methodology …………................................................................................ 10
   3.1 Methodology Overview ............................................................................ 10
       3.1.1 Requirement Analysis ................................................................. 11
       3.1.2 Design ......................................................................................... 12
       3.1.3 Implementation ........................................................................... 13
       3.1.4 Testing ........................................................................................ 14
   3.2 Requirement Analysis ............................................................................. 15
   3.3 Design Phase ........................................................................................... 16
      3.3.1 System Architecture ................................................................... 17
      3.3.2 Hardware Layout ........................................................................ 18
      3.3.3 System Workflow ....................................................................... 19
   3.4 Implementation Phase ............................................................................ 20
   3.5 Testing Phase ...................................................................................................... 22

- Software Requirement Specifications ……………................................................... 23
   4.1 Functional Requirements ...................................................................... 23
   4.2 Non-Functional Requirements ............................................................. 24
   4.3 Software and Hardware Requirements ............................................... 25
       4.3.1 Software Requirements ............................................................ 26
       4.3.2 Hardware Requirements ........................................................... 26
   4.4 Constraints ............................................................................................. 27
   4.5 Future Enhancements ............................................................................ 28

- Discussion and Conclusion …………........................................................................ 29
   5.1Discussion ………….............................................................................................29
   5.2 Conclusion ...........................................................................................................30

- References ................................................................................................................... 31

- Appendices…………..................................................................................................32
      Appendix A: Project Schedule (Gantt Chart) ................................................……32

#### LIST OF TABLES

TABLE 	TITLE 	PAGE

Table 1:      	               Proposed Enhacements 	23

#### LIST OF FIGURES

FIGURE 	TITLE 	PAGE

Figure 1:                  Car working flowchart with Car pic  	17

LIST OF APPENDICES

| APPENDIX | TITLE | PAGE |
| --- | --- | --- |
| APPENDIX A: | Gantt Chart | 28 |

CHAPTER 1 INTRODUCTION:

### 1.1 Background:

With rapid advancements in automation and robotics, autonomous vehicles have become a focus of modern engineering. The Line Following Car project explores foundational principles of autonomous navigation, enabling a vehicle to follow a predefined path with minimal human intervention. Using IR sensors for path detection and a microcontroller for decision-making, this project offers practical insights into the integration of hardware and software. It represents an essential step toward understanding real-world applications in autonomous systems, such as self-driving cars, industrial robots, and automated logistics.

### 1.2 Problem Statements:

Developing systems that can reliably follow a path and adapt to dynamic conditions remains a fundamental challenge in robotics. Effective line-following navigation demands precision in detecting the path and real-time adjustments to maintain accuracy. Additionally, achieving seamless communication between sensors, microcontrollers, and motors requires a robust and well-integrated system design, which is often a complex task. Precise and timely decision-making is critical for avoiding errors in navigation, as any delay or inaccuracy can lead to deviations or operational failures. Furthermore, implementing efficient motor control algorithms using assembly language or other low-level programming methods presents both a technical and educational challenge, as it requires a deep understanding of hardware-software interaction and optimization techniques. Together, these challenges form the core obstacles in building a reliable and functional line-following car system.

### 1.3 Aims and Objectives:

- Design and implement a car capable of autonomously following a predefined black line on a white surface.

- Integrate IR sensors to detect and respond to variations in the path.

- Develop motor control algorithms to enable precise movement and direction adjustments.

- Enhance problem-solving skills through real-world challenges in robotics and automation.

- Gain practical experience in hardware-software integration using microcontrollers and sensor technologies.

### 1.4 Scope of Project:

The scope of the Line Following Car project includes the design, implementation, and testing of a functional autonomous vehicle prototype. Key features and areas of focus include:

- Core Functionalities:

  - Accurate line detection using IR sensors.

  - Autonomous navigation with real-time adjustments to path changes.

  - Motor control to ensure smooth and precise operation.

- Technical Approach:

  - Integration of an STM32 microcontroller to process sensor inputs and execute motor commands.

  - Assembly language programming for efficient motor control algorithms.

  - Modular hardware design to simplify future enhancements and scalability.

- Target Audience:

  - Engineering students and researchers aiming to explore autonomous navigation.

  - Industries interested in foundational technologies for robotics and automation.

- Limitations:

  - Initial prototype focuses on basic line-following functionality, with limited environmental adaptability.

  - Expansion to more complex navigation or obstacle avoidance is outside the immediate scope of this project.

This project serves as a stepping stone to deeper exploration in autonomous robotics and provides essential skills for real-world applications in automation and navigation systems.

CHAPTER 2

## LITERATURE REVIEW

### 2.1: Automation in Navigation Systems

Automation in navigation systems has revolutionized industries, from logistics to autonomous vehicles. Line-following robots are a foundational example of autonomous systems, enabling real-time navigation along a predefined path. These systems rely on sensors and microcontrollers to detect and process the path's attributes, which can be applied to larger-scale technologies such as self-driving cars and automated transport systems. Studies highlight their potential in simplifying processes like material handling, industrial automation, and delivery systems.

### 2.2: Role of Sensors in Line Following Cars

Sensors, particularly Infrared (IR) sensors, play a crucial role in line-following systems. These sensors detect contrasts in the surface, such as a black line on a white background, and provide input to the microcontroller for decision-making. Literature emphasizes the importance of precise sensor placement and calibration to ensure accurate path detection. IR sensors, combined with algorithms for filtering noise, enable robust performance even in varying lighting conditions.

### 2.3: Microcontrollers and Real-Time Control

Microcontrollers, such as the STM32, serve as the central processing units in line-following cars. They process sensor data, execute algorithms, and control motor actions. Real-time control is critical to ensure the car stays on track and adapts quickly to path variations. Research shows that efficient microcontroller programming can significantly enhance the system's responsiveness and reliability. Low-level programming languages, like Assembly, are often employed for motor control to achieve high precision and minimal latency.

### 2.4: Challenges in Building Line Following Cars

Despite their widespread application, developing line-following cars comes with challenges. Seamless hardware-software integration is a key hurdle, as it requires effective communication between sensors, microcontrollers, and actuators. Additionally, ensuring consistent performance in dynamic conditions, such as curved paths or uneven surfaces, adds complexity. Studies also highlight the difficulty in optimizing motor control algorithms, which must strike a balance between accuracy and speed.

### 2.5: Innovations and Future Trends

Recent advancements in robotics have introduced improvements in line-following systems, such as integrating machine learning algorithms for adaptive path-following. Innovations in sensor technology, including the use of multiple sensors for redundancy, have enhanced reliability. Future trends indicate a shift toward more complex navigation systems that combine line-following capabilities with obstacle avoidance and decision-making features.

This literature review provides a comprehensive understanding of the technologies, challenges, and advancements related to line-following cars, setting the foundation for developing a robust and efficient prototype.

CHAPTER 3

PROJECT METHODOLOGY

The project methodology outlines the systematic approach adopted to design, develop, and implement the Line Following Car. It includes planning, analysis, design, development, and testing phases to ensure a structured and efficient workflow.

3.1 Methodology Overview:

The development of the Line Following Car follows the Waterfall Model to maintain clarity in progression between phases and minimize overlap. Each phase is completed and reviewed before proceeding to the next.

    - Requirement Analysis:

  - Understanding the system's needs, such as line detection, autonomous navigation, and motor control.

  - Identifying the scope and constraints, such as limited environmental adaptability in the prototype phase.

    - Design:

  - Designing the system architecture, including sensor placement, microcontroller functionality, and motor control.

  - Creating workflow diagrams to visualize the interactions between hardware and software components.

    - Implementation:

  - Coding the system using Assembly language for motor control and high-level programming for sensor integration.

  - Leveraging hardware components like IR sensors, an STM32 microcontroller, and DC motors.

    - Testing:

  - Performing functional and non-functional testing to identify and resolve errors.

  - Validating the performance of each module independently and as part of the complete system.

3.2 Requirement Analysis:

The requirement analysis phase focused on understanding system needs and translating them into actionable goals. Key steps include:

    - Stakeholder Consultation:

Consulting with instructors and peers to define the primary features, such as autonomous line detection and navigation.

    - Feasibility Study:

Evaluating the feasibility of implementing features like IR sensor integration and precise motor control algorithms within the project's scope.

    - Use Case Development:

Creating use cases for core functionalities, including line detection, obstacle handling, and motor movement.

3.3 Design Phase:

The design phase involved creating the architectural blueprints for the system.

3.3.1 System Architecture:

- Sensor Integration: IR sensors detect a black line on a white surface and send signals to the microcontroller.

- Microcontroller:The STM32 processes sensor inputs and calculates motor adjustments.

- Motor Control:Motors are controlled using pulse width modulation (PWM) for precise movement and speed adjustments.

    - Hardware Layout:

- Sensors are placed at the front of the car for optimal line detection.

- The microcontroller connects to the motors and sensors through GPIO.

3.3.3 System Workflow:

- Line Detection: IR sensors detect the position of the line and send data to the microcontroller.

- Path Adjustment:The microcontroller adjusts motor speeds to correct the car’s direction.

- Navigation:The car continues to follow the line based on real-time feedback from the sensors.

  - Implementation Phase:

The implementation phase focused on coding and assembling hardware components. Key activities included:

    - Backend Development:

  - Using Assembly language for motor control and C for sensor integration and decision-making.

  - Implementing efficient algorithms to process sensor data and control motor actions.

      - Assembly Code:

.syntax unified

.cpu cortex-m3

.thumb

.global Reset_Handler

.global main

.section .text

Reset_Handler:

LDR   SP, =_estack       // Initialize stack pointer

BL    main

B     .

main:

// Enable clocks for GPIOA and GPIOB

LDR   R0, =0x40021018     // RCC_APB2ENR

LDR   R1, [R0]

ORR   R1, R1, #(1 << 2)   // IOPAEN

ORR   R1, R1, #(1 << 3)   // IOPBEN

STR   R1, [R0]

// Configure PA3-PA6, PA8-PA11 as Output Push-Pull (motors and ultrasonic TRIG)

LDR   R0, =0x40010800     // GPIOA_CRL

MOV   R1, #0x22222222     // MODE = 0b10 (Output), CNF = 0b00 (Push-pull)

STR   R1, [R0]

LDR   R0, =0x40010804     // GPIOA_CRH (PA8-PA11)

MOV   R1, #0x22222222

STR   R1, [R0]

// Configure PB0, PB1 as Input floating (IR sensors)

LDR   R0, =0x40010C00     // GPIOB_CRL

MOV   R1, #0x44444444

STR   R1, [R0]

BL    loop

loop:

// Read ultrasonic distance

BL    getDistance

// Compare distance <= 10

CMP   R0, #10

BLE   stopMotors

// Read IR sensors PB0 and PB1

LDR   R1, =0x40010C08     // GPIOB_IDR

LDR   R2, [R1]

ANDS  R3, R2, #1          // LEFT_IR = PB0

ANDS  R4, R2, #2          // RIGHT_IR = PB1

CMP   R3, #0

BNE   check_left_right

CMP   R4, #0

BNE   check_left_right

BL    moveForward

B     loop

check_left_right:

CMP   R3, #0

BNE   check_right

CMP   R4, #2

BNE   stopMotors

BL    turnRight

B     loop

check_right:

CMP   R3, #1

BNE   stopMotors

CMP   R4, #0

BNE   stopMotors

BL    turnLeft

B     loop

stopMotors:

LDR   R0, =0x4001080C     // GPIOA_BSRR

LDR   R1, =0x00F00000     // Clear IN1-IN4 (PA3â€“PA6)

STR   R1, [R0]

LDR   R0, =0x40010810     // GPIOA_ODR

LDR   R1, [R0]

BIC   R1, R1, #(1 << 8)   // Clear ENA (PA8)

BIC   R1, R1, #(1 << 9)   // Clear ENB (PA9)

STR   R1, [R0]

BX    LR

moveForward:

LDR   R0, =0x4001080C

LDR   R1, =((1<<2)|(1<<4)|(1<<(3+16))|(1<<(5+16))) // IN2 & IN3 HIGH, IN1 & IN4 LOW

STR   R1, [R0]

BX    LR

turnLeft:

LDR   R0, =0x4001080C

LDR   R1, =((1<<3)|(1<<5)|(1<<(2+16))|(1<<(4+16))) // IN1 & IN3 HIGH, IN2 & IN4 LOW

STR   R1, [R0]

BX    LR

turnRight:

LDR   R0, =0x4001080C

LDR   R1, =((1<<2)|(1<<4)|(1<<(3+16))|(1<<(5+16))) // IN2 & IN4 HIGH, IN1 & IN3 LOW

STR   R1, [R0]

BX    LR

getDistance:

// Send TRIG pulse on PA10

LDR   R0, =0x4001080C     // GPIOA_BSRR

LDR   R1, =(1 << 10)

STR   R1, [R0]

BL    delay10us

LDR   R1, =(1 << (10 + 16))

STR   R1, [R0]

// Wait for ECHO high on PA11

wait_echo_high:

LDR   R0, =0x40010808     // GPIOA_IDR

LDR   R1, [R0]

TST   R1, #(1 << 11)

BEQ   wait_echo_high

// Measure duration while ECHO is high (basic loop count)

MOV   R2, #0

wait_echo_low:

LDR   R1, [R0]

TST   R1, #(1 << 11)

BEQ   done_measure

ADD   R2, R2, #1

CMP   R2, #30000

BHI   done_measure

B     wait_echo_low

done_measure:

MOV   R0, R2

BX    LR

delay10us:

MOV   R2, #720          // Approx 10us delay

loop_delay:

SUBS  R2, R2, #1

BNE   loop_delay

BX    LR

    - Hardware Integration:

  - The STM32F103C8T6 microcontroller was used as the central controller to manage motor control, sensor data processing, and obstacle detection.IR sensors were calibrated to detect contrasts on the surface (black line vs white background) and provide digital output signals.

  - The ultrasonic sensor (HC-SR04) was mounted at the front of the car to detect obstacles within a range of up to 10 cm.

  - The STM32 microcontroller was programmed to interpret IR and ultrasonic sensor data and make real-time movement decisions.

  - The L298N motor driver was used to control two DC motors, enabling forward motion, left turns, right turns, and stopping.

  - GPIO pins were assigned to control motor direction (IN1–IN4) and speed (ENA/ENB) using software-based PWM.A common ground was established among the STM32 board, sensor modules, and motor driver to ensure signal integrity.

  - Motors were powered by an external 9–12V source connected to the L298N, while the STM32 and sensors were powered via 5V.

  - Sensors were positioned at the front of the car: IR sensors were placed at the bottom for line tracking; the ultrasonic sensor faced forward for obstacle detection.

  - All wiring was secured and organized to prevent disconnections and minimize electrical noise during operation.

CIRCUIT DIAGRAM:

    - Motor Control:

- PWM signals were used to control motor speeds for smooth turns and straight-line motion.

3.5 Testing Phase:

Testing ensured that the system met both functional and non-functional requirements. The process included:

    - Unit Testing:

  - Individual modules (e.g., sensor readings, motor control) were tested for accuracy.

  - Focused on ensuring proper signal processing and response times.

    - Integration Testing:

  - The complete system was tested to ensure smooth interactions between sensors, the microcontroller, and motors.

  - Validated real-time adjustments during navigation.

    - Performance Testing:

  - The car was tested on various path shapes (e.g., straight lines, curves) and under different lighting conditions to evaluate adaptability and reliability.

By following this structured methodology, the Line Following Car project ensured a systematic and efficient approach to achieving its objectives.

CHAPTER 4:

SOFTWARE REQUIREMENT SPECIFICATIONS

#### 4.1 Functional Requirements:

Functional requirements describe the specific behavior or functions of the system. These ensure that the line-following car operates as intended to meet project goals.

    - Line Detection:

  - The system must use IR sensors to detect and follow a black line on a white surface.

  - Sensors should accurately identify deviations in the path and communicate these to the microcontroller.

    - Obstacle Handling:

  - The car should pause if an obstacle is detected in its path, resuming operation once cleared.

  - Sensors should differentiate between obstacles and the path boundary.

    - Motor Control:

  - Motors should adjust speed dynamically for smooth navigation of curves and straight paths.

  - The system should allow for precise control of left and right motor speeds.

    - Real-Time Decision Making:

  - The microcontroller must process sensor data in real-time to adjust motor actions immediately.

  - Ensure consistent navigation even with path irregularities like sharp turns or intersections.

#### 4.2 Non-Functional Requirements:

Non-functional requirements define the operational constraints, focusing on usability, reliability, and performance.

    - Performance:

  - Ensure a response time of less than 100ms for sensor data processing and motor adjustments.

  - The car should navigate curves and straight paths without deviation from the line.

    - Usability:

  - Provide an easily accessible interface for calibrating sensors and testing motor performance.

  - Maintain simple assembly instructions for ease of replication.

    - Portability:

  - The system should operate on a lightweight chassis for easy movement and testing.

  - Components should be compatible with standard microcontroller development environments.

    - Reliability:

  - The car should operate consistently under varying lighting conditions.

  - Sensors must provide accurate readings on both high-contrast and slightly worn paths.

    - Scalability:

  - Design a modular system to allow future integration of features like obstacle avoidance or multiple path options.

#### 4.3 Software and Hardware Requirements:

4.3.1 Software Requirements:

- Development Tools:

  - STM32CubeIDE: For writing and debugging microcontroller code.

  - Simulation Software: Tools like Proteus for testing sensor and motor integration virtually.

- Programming Language and Framework:

  - Assembly language for motor control.

  - C/C++ for processing sensor data and system logic.

- External Libraries:

  - IR sensor calibration libraries for better accuracy.

  - PWM control libraries for precise motor speed regulation.

4.3.2 Hardware Requirements:

- Minimum Requirements:

  - Processor: STM32 microcontroller.

  - Sensors: IR sensors for path detection.

  - Motors: DC motors with PWM control capability.

  - Power Supply: 7.4V LiPo battery or equivalent.

- Recommended Requirements:

  - Microcontroller: STM32F4 series for better processing speed.

  - Sensors: Array of IR sensors for enhanced detection accuracy.

  - Motors: High-torque DC motors for smoother navigation.

#### 4.4 Constraints:

- The car can only follow predefined black lines on a white surface and cannot adapt to complex paths or other colors.

- Performance may vary under poor lighting conditions or worn-out surfaces.

- The prototype is limited to basic line-following functionality and does not include obstacle avoidance or advanced navigation features.

- Battery life may limit the duration of continuous operation.

#### 4.5 Future Enhancements:

- Advanced Navigation Features:

  - Integrate obstacle detection and avoidance capabilities.

  - Enable multi-path navigation with programmable path selection.

- Improved Sensor System:

  - Replace basic IR sensors with an array or camera-based line detection for enhanced accuracy.

- Wireless Control and Monitoring:

  - Include Bluetooth or Wi-Fi modules to control the car remotely and monitor performance in real time.

- Power Efficiency:

  - Optimize battery usage with energy-efficient motor control algorithms.

- Environment Adaptability:

  - Develop functionality for following lines of different colors or patterns.

| No. | Category | Description |
| --- | --- | --- |
| 1 | Advanced Navigation | Obstacle detection and multi-path navigation. |
| 2 | Sensor System | Camera-based line detection. |
| 3 | Wireless Control | Remote control and real-time monitoring. |
| 4 | Power Efficiency | Improved battery life and motor efficiency. |
| 5 | Environment Adaptability | Line-following for various colors and patterns. |

TABLE 01: PROPOSED ENHANCEMENTS

CHAPTER 5:

DISCUSSION AND CONCLUSION Discussion:

The Line Following Car project successfully demonstrates the integration of fundamental robotics principles, including sensor-based navigation, real-time decision-making, and motor control. The project relied on IR sensors to detect and follow a predefined path, which allowed for efficient and consistent navigation. The use of the STM32 microcontroller enabled precise processing of sensor data and seamless communication with motors for dynamic adjustments. This approach highlights the importance of hardware-software integration in robotics and automation.

During development, key challenges included calibrating sensors for accurate line detection under varying lighting conditions and implementing algorithms for smooth motor control. These were addressed through iterative testing and refinement, ensuring the system responded effectively to path deviations. The project also provided hands-on experience in programming with Assembly and C/C++, further reinforcing the understanding of low-level programming in real-world applications.

While the car performed reliably on simple paths, limitations were observed in handling sharp turns and intersections. The absence of obstacle avoidance features highlights an area for potential improvement. Despite these constraints, the project achieves its primary goal of exploring autonomous navigation, laying the groundwork for more advanced applications in robotics.

#### Conclusion

The Line Following Car project successfully fulfills its objective of developing an autonomous vehicle capable of following a predefined path using IR sensors and a microcontroller. By implementing real-time decision-making algorithms and precise motor control, the system demonstrates the feasibility of simple robotic navigation. The project highlights the practical applications of sensor technologies and low-level programming in robotics, providing a solid foundation for future advancements.

This project not only enhances understanding of automation principles but also emphasizes the importance of system design, testing, and iterative development in achieving reliable results. Future enhancements, such as incorporating obstacle avoidance, advanced sensors, and wireless control, will further extend the capabilities of this system, making it suitable for more complex environments and applications. Overall, the project serves as a valuable learning experience and a stepping stone towards more sophisticated autonomous robotic systems.

REFERENCES:

 Books and Manuals:

- B. Kumar and R. Joshi, Introduction to Robotics and Automation. Springer, 2021.

- S. F. Barrett and D. J. Pack, Embedded Systems: Design and Applications with the STM32 Microcontroller. CRC Press, 2016.

 Research Articles:

- R. Gupta and S. Patel, "Autonomous Line Following Robots: Challenges and Applications," International Journal of Robotics Research, vol. 15, no. 2, pp. 145-159, 2020.

- C. Y. Lee and K. T. Wong, "IR Sensor Calibration Techniques for Line Following Robots," IEEE Sensors Journal, vol. 18, no. 8, pp. 2307-2314, 2019.

Online Resources:

- Arduino Project Hub, "Building a Line Following Robot Using IR Sensors," 2022. [Online]. Available: https://create.arduino.cc.

- STM32 Documentation, "Programming and Development with STM32 Microcontrollers," 2023. [Online]. Available: https://www.st.com.

APPENDICES

APPENDIX A: Project Schedule

I. 	Gantt Chart